1.  DHT11 with Arduino.
#include <dht11.h>

#define DHT11PIN 4

dht11 DHT11;

void  setup()

{

Serial.begin(9600);

}

void loop()

{

Serial.println();

int chk = DHT11.read(DHT11PIN);

Serial.print("Humidity (%): ");

Serial.println((float)DHT11.humidity, 2);

Serial.print("Temperature  (C): ");

Serial.println((float)DHT11.temperature, 2);

delay(2000);

}

https://projecthub.arduino.cc/arcaegecengiz/using-dht11-12f621

2. Traffic Light using Arduino

int red = 9;

int yellow = 8;

int green = 7;

void setup(){

pinMode(red, OUTPUT);

pinMode(yellow, OUTPUT);

pinMode(green,  OUTPUT);

}

void loop(){

digitalWrite(red, HIGH);

delay(15000);

digitalWrite(red,  LOW);

digitalWrite(yellow, HIGH);

delay(1000);

digitalWrite(yellow,  LOW);

delay(500);

digitalWrite(yellow, HIGH);

delay(1000);

digitalWrite(yellow,  LOW);

delay(500);

digitalWrite(yellow, HIGH);

delay(1000);

digitalWrite(yellow,  LOW);

delay(500);

digitalWrite(yellow, HIGH);

delay(1000);

digitalWrite(yellow, LOW);

delay(500);

digitalWrite(yellow, HIGH);

delay(1000);

digitalWrite(yellow, LOW);

delay(500);

digitalWrite(green, HIGH);

delay(20000);

digitalWrite(green,  LOW);

//

digitalWrite(yellow, HIGH);

delay(1000);

digitalWrite(yellow,  LOW);

delay(500);

digitalWrite(yellow, HIGH);

delay(1000);

digitalWrite(yellow,  LOW);

delay(500);

digitalWrite(yellow, HIGH);

delay(1000);

digitalWrite(yellow, LOW);

delay(500);

digitalWrite(yellow, HIGH);

delay(1000);

digitalWrite(yellow, LOW);

delay(500);

digitalWrite(yellow, HIGH);

delay(1000);

digitalWrite(yellow, LOW);

delay(500);

}

https://projecthub.arduino.cc/agarwalkrishna3009/traffic-light-using-arduino-a-beginner-project-35f8c6

3. Ultrasonic Sensor Distance Radar

#include <Servo.h>

/*

code for arduino bord ultrasonic radar system.

for more arduino projects --->  teamdiy.in

*/

Servo leftRightServo;         // set a variable to map the servo

int leftRightPos = 0;         // set a variable to store the servo position

const int numReadings = 10;   // set a variable for the number of readings to take

int index = 0;                // the index of the current reading

int total = 0;                // the total of all readings

int average = 0;              // the average

int echoPin = 7;              // the SRF05's echo pin

int initPin = 8;              // the SRF05's init pin

unsigned long pulseTime = 0;  // variable for reading the pulse

unsigned long distance = 0;   // variable for storing distance

/* setup the pins, servo and serial port */

void setup() {

leftRightServo.attach(9);

// make the init pin an output:

pinMode(initPin, OUTPUT);

// make the echo pin an input:

pinMode(echoPin, INPUT);

// initialize the serial port:

Serial.begin(9600);

}

/* begin rotating the servo and getting sensor values */

void loop() {

for(leftRightPos = 0; leftRightPos < 180; leftRightPos++) {  // going left to right.

leftRightServo.write(leftRightPos);

for(index = 0; index<=numReadings;index++) // take x number of readings from the sensor and average them

digitalWrite(initPin, LOW);

delayMicroseconds(50);

digitalWrite(initPin, HIGH);// send signal

delayMicroseconds(50);             // wait 50 microseconds for it to return

digitalWrite(initPin, LOW);                            // close signal

pulseTime = pulseIn(echoPin, HIGH);      // calculate time for signal to return

distance = pulseTime/58;                               // convert to centimetres

total = total + distance;                              // update total

delay(10);

}

average = total/numReadings;                // create average reading

if (index >= numReadings){// reset the counts when at the last item of the array

index = 0;

total = 0;

}

Serial.print("X");     // print leading X to mark the following value as degrees

Serial.print(leftRightPos);                   // current servo position

Serial.print("V");           // preceeding character to separate values

Serial.println(average);            // average of sensor readings

}

/*

start going right to left after we got to 180 degrees

same code as above

*/

for(leftRightPos = 180; leftRightPos > 0; leftRightPos--) {  // going right to left

leftRightServo.write(leftRightPos);

for (index = 0; index<=numReadings;index++) {

digitalWrite(initPin, LOW);

delayMicroseconds(50);

digitalWrite(initPin, HIGH);

delayMicroseconds(50);

digitalWrite(initPin, LOW);

pulseTime = pulseIn(echoPin, HIGH);

distance = pulseTime/58;

total = total + distance;

delay(10);

}

average = total/numReadings;

if (index >= numReadings)  {

index = 0;

total = 0;

}

Serial.print("X");

Serial.print(leftRightPos);

Serial.print("V");

Serial.println(average);

}

}

https://www.hackster.io/Manikantsavadatti/diy-ultrasonic-radar-system-1f4d1c

4. Arduino Trash Can

#include<Servo.h>

Servo servo;

int const trigPin = 6;

int const   echoPin = 5;

void setup()

{

pinMode(trigPin, OUTPUT);

pinMode(echoPin,   INPUT);

servo.attach(3);

}

void loop()

{       int duration, distance;

digitalWrite(trigPin,   HIGH);

delay(1);

digitalWrite(trigPin, LOW);

// Measure the pulse input   in echo pin

duration = pulseIn(echoPin, HIGH);

// Distance is half the duration   devided by 29.1 (from datasheet)

distance = (duration/2) / 29.1;

// if distance   less than 0.5 meter and more than 0 (0 or less means over range)

if (distance   <= 50 && distance >= 0) {

servo.write(50);

delay(3000);

} else {

servo.write(160);

}

//   Waiting 60 ms won't hurt any one

delay(60);

}

https://projecthub.arduino.cc/ashraf_minhaj/arduino-trash-bot-auto-openclose-trash-bin-c81eff

5. Soil Moisture Sensor

// Moisture Sensor Arduino Code

//By Circuitdigest

#define ledPin 6

#define sensorPin A0

void setup() {

Serial.begin(9600);

pinMode(ledPin, OUTPUT);

digitalWrite(ledPin, LOW);

}

void loop() {

Serial.print("Analog output: ");

Serial.println(readSensor());

delay(500);

}

//  This function returns the analog data to calling function

int readSensor() {

int sensorValue = analogRead(sensorPin);  // Read the analog value from sensor

int outputValue = map(sensorValue, 0, 1023, 255, 0); // map the 10-bit data to 8-bit data

analogWrite(ledPin, outputValue); // generate PWM signal

return outputValue;             // Return analog moisture value

}

https://circuitdigest.com/microcontroller-projects/interfacing-soil-moisture-sensor-with-arduino-uno

6. CONTROLLED LEDS USING ARDUINO AND Push Button

void setup() {

pinMode(12,OUTPUT);

pinMode(2,INPUT);

}

void loop() {

if(digitalRead(2) == HIGH){

digitalWrite(12,HIGH);

}

else{

digitalWrite(12,LOW);

}

}

https://www.youtube.com/watch?v=yBgMJssXqHY&t=1121s

7. ARDUINO AND MQ-135 Air Quality Monitoring

#include <LiquidCrystal_I2C.h>

#include "MQ135.h"

#define ANALOGPIN A2    //  Define Analog PIN on Arduino Board

#define RZERO 206.85    //  Define RZERO Calibration Value

MQ135 gasSensor = MQ135(ANALOGPIN);

LiquidCrystal_I2C lcd(0x27, 16,2);

void setup()

{

lcd.init();

lcd.begin(16,2);//Defining 16 columns and 2 rows of lcd display

lcd.backlight();

Serial.begin(9600);

float rzero = gasSensor.getRZero();

delay(3000);

Serial.print("MQ135 RZERO Calibration Value : ");

Serial.println(rzero);

}

void loop() {

float ppm = gasSensor.getPPM();

delay(1000);

Serial.print("CO2 ppm value : ");

Serial.println(ppm);

lcd.setCursor(0,0);

lcd.print("   Tech Maker   ");

lcd.setCursor(0,1);

lcd.print("CO2 : ");

lcd.print(ppm);

lcd.print("PPM");

}

https://electronicsprojectshub.com/interfacing-mq-135-gas-sensor-with-arduino/

8. ARDUINO BASED MOTION DETECTION USING PIR SENSOR

const int pirPin = 5;  // Digital pin connected to the PIR sensor

void setup() {

pinMode(pirPin, INPUT);

pinMode(7, OUTPUT);  // Set PIR sensor pin as input

Serial.begin(9600);      // Initialize serial communication for debugging (optional)

delay(5000);

}

void loop() {

int pirState = digitalRead(pirPin); // Read PIR sensor state (HIGH or LOW)

if (pirState == HIGH) {

Serial.println("Motion detected!");

delay(5000); // Delay for 1 second (adjust as needed)

// Motion detected

digitalWrite(7, HIGH);

delay(500);

} else {

// No motion

Serial.println("No motion detected.");

delay(5000);

digitalWrite(7,LOW);

delay(500);

}

}

https://circuitdigest.com/microcontroller-projects/interface-pir-sensor-with-arduino

9. Rain Detection System using Arduino and Rain Sensor

#define rainfall A0

#define buzzer 5

int value;

int set=10;

void setup() {

Serial.begin(9600);

pinMode(buzzer,OUTPUT);

pinMode(rainfall,INPUT);

}

void loop() {

value = analogRead(rainfall);

Serial.println("LOL");

Serial.println(value);

value = map(value,0,1023,225,0);

Serial.println(value);

if(value>=set){

Serial.println("rain detected");

digitalWrite(buzzer,HIGH);

}

else{

digitalWrite(buzzer,LOW);

}

delay(200);

}

https://circuitdigest.com/microcontroller-projects/rain-detector-using-arduino

10. Audio Frequency Detector

#include  "arduinoFFT.h"

#define SAMPLES 128       //SAMPLES-pt FFT. Must  be a base 2 number. Max 128 for Arduino Uno.

#define SAMPLING_FREQUENCY 2048  //Ts = Based on Nyquist, must be 2 times the highest expected frequency.

arduinoFFT  FFT = arduinoFFT();

unsigned int samplingPeriod;

unsigned long microSeconds;

double vReal[SAMPLES]; //create vector of size SAMPLES to hold real values

double  vImag[SAMPLES]; //create vector of size SAMPLES to hold imaginary values

void  setup()

{

Serial.begin(115200); //Baud rate for the Serial Monitor

samplingPeriod = round(1000000*(1.0/SAMPLING_FREQUENCY)); //Period in microseconds

}

void loop()

{

/*Sample SAMPLES times*/

for(int  i=0; i<SAMPLES; i++)

{

microSeconds = micros();    //Returns the  number of microseconds since the Arduino board began running the current script.

vReal[i] = analogRead(0); //Reads the value from analog pin  0 (A0), quantize it and save it as a real term.

vImag[i] = 0; //Makes  imaginary term 0 always

/*remaining wait time between samples if  necessary*/

while(micros() < (microSeconds + samplingPeriod))

{

//do nothing

}

}

/*Perform FFT on samples*/

FFT.Windowing(vReal, SAMPLES, FFT_WIN_TYP_HAMMING, FFT_FORWARD);

FFT.Compute(vReal,  vImag, SAMPLES, FFT_FORWARD);

FFT.ComplexToMagnitude(vReal, vImag, SAMPLES);

/*Find peak frequency and print peak*/

double peak = FFT.MajorPeak(vReal,  SAMPLES, SAMPLING_FREQUENCY);

Serial.println(peak);     //Print out the most  dominant frequency.

/*Script stops here. Hardware reset required.*/

while (1); //do one time

}

https://projecthub.arduino.cc/lbf20012001/audio-frequency-detector-d300e3

11.  ARDUINO PIANO

#define NOTE_C 262

#define NOTE_D 294

#define NOTE_E 330

#define NOTE_A 440

#define NOTE_B 493

#define ACTIVATED LOW

const int PIEZO = 11;

const int BUTTON_C = 10;

const int BUTTON_D = 9;

const int BUTTON_E = 6;

const int BUTTON_A = 5;

const int BUTTON_B = 4;

void setup()

{

pinMode(BUTTON_C, INPUT);

digitalWrite(BUTTON_C,HIGH);

pinMode(BUTTON_D, INPUT);

digitalWrite(BUTTON_D,HIGH);

pinMode(BUTTON_E, INPUT);

digitalWrite(BUTTON_E,HIGH);

pinMode(BUTTON_A, INPUT);

digitalWrite(BUTTON_A,HIGH);

pinMode(BUTTON_B, INPUT);

digitalWrite(BUTTON_B,HIGH);

}

void loop()

{

while(digitalRead(BUTTON_C) == ACTIVATED)

{

tone(PIEZO,NOTE_C);

}

while(digitalRead(BUTTON_D) == ACTIVATED)

{

tone(PIEZO,NOTE_D);

}

while(digitalRead(BUTTON_E) == ACTIVATED)

{

tone(PIEZO,NOTE_E);

}

while(digitalRead(BUTTON_A) == ACTIVATED)

{

tone(PIEZO,NOTE_A);

}

while(digitalRead(BUTTON_B) == ACTIVATED)

{

tone(PIEZO,NOTE_B);

}

noTone(PIEZO);

}

https://techatronic.com/arduino-based-piano-using-push-buttons-arduino-mini-projects/

12. ARDUINO BASED SPEECH TO TEXT

#include "Talkie.h"

#include "Vocab_US_Large.h"

#include "Vocab_Special.h"

Talkie voice;

void setup() {

}

void loop() {

voice.say(spPAUSE2);

voice.say(sp2_DANGER);

voice.say(sp2_DANGER);

voice.say(sp3_STORM);

voice.say(sp3_IN);

voice.say(sp3_THE);

voice.say(sp3_NORTH);

}

https://circuitdigest.com/microcontroller-projects/arduino-based-text-to-speech-converter

13. Automatic Water Dispenser using Arduino

#define trigger 9
#define echo 8
#define LED 13
#define MOSFET 12

float time=0,distance=0;

void setup()

{

Serial.begin(9600);

pinMode(trigger,OUTPUT);
pinMode(echo,INPUT);

pinMode(LED,OUTPUT);

pinMode(MOSFET,OUTPUT);

delay(2000);
}
void loop()

{
measure_distance();

if(distance<10)
{
digitalWrite(LED,HIGH);digitalWrite(MOSFET,HIGH);

}

else

{

digitalWrite(LED,LOW);digitalWrite(MOSFET,LOW);

}

delay(500);
}

void measure_distance()

{
digitalWrite(trigger,LOW);

delayMicroseconds(2);

digitalWrite(trigger,HIGH);

delayMicroseconds(10);

digitalWrite(trigger,LOW);

delayMicroseconds(2);
time=pulseIn(echo,HIGH);
distance=time*340/20000;

}

https://circuitdigest.com/microcontroller-projects/arduino-automatic-water-dispenser